RC Over Current Protection Fault

Alarm Attribute

| Alarm name | Alarm Severity | Alarm Category |
| --- | --- | --- |
| RC Over Current Protection Fault | General Alarm | Inverter alarm |

Possible Cause

| Cause ID | Possible Cause |
| --- | --- |
| 1 | Residual current over-current Fault: On-grid residual current exceeds the threshold set by the safety code. |

Suggestion

1.Check the error frequency. If the error occurs occasionally, inspect the PE wire to ensure it is properly connected and not damaged.

2.If the issue remains, provide the inverter's SN number to the SolaX technical support team for further analysis.