BMS Hardware Protect

Alarm Attribute

| Alarm name | Alarm Severity | Alarm Category |
| --- | --- | --- |
| BMS Hardware Protect | General | Battery alarm |

Possible Cause

| Cause ID | Possible Cause |
| --- | --- |
| 1 | Slave battery hardware fault: the hardware of the slave battery has problem |

Suggestion

1.The fault commonly occurs during the battery charging and discharging process, and is usually accompanied by errors related to the cell temperature or voltage. First, let the battery sit for 30 minutes. Then, Based on the accompanying error, use the T-BAT-Charger to check the detailed cell voltage and temperature information of each battery, Identify the faulty battery.
2.If the T-BAT-Charger is unavailable, provide the SN number of the inverter to SolaX technical support team.