Cluster Communication Quantity Mismatch

Alarm Attribute

| Alarm name | Alarm Severity | Alarm Category |
| --- | --- | --- |
| Cluster Communication Quantity Mismatch | General | Battery alarm |

Possible Cause

| Cause ID | Possible Cause |
| --- | --- |
| 1 | The number of battery cluster communications does not match. |
| 2 | There is a communication problem between the BMS and the parallel box. |

Suggestion

1. Check whether the communication cable between the BMS and the parallel box is connected correctly.

2. Turn on the main control battery in turn, wait for the self-test to complete, and then start the battery parallel box.

3.If the issue remains,provide the inverter's SN number to the SolaX technical support team for further analysis.